Table 9 Type of Driven Pile Comparison

| Pile Driving Method | Strengths | Weakness |
| --- | --- | --- |
| Hammer Pile | Fast and economical for large projects in open areas or areas free from building density. | High noise and vibration pose a risk to the surrounding buildings and the comfort of the construction environment. |
| Hydraulic static pile driver (HSPD) | Minimal vibration and noise, suitable for densely built-up or populated areas. | Relatively higher costs, piles must be of higher quality to withstand compressive forces. |
| Vibratory pile driver | The installation of the tool is relatively faster under soil conditions that meet the tool criteria. | Less effective on hard clay soil, still causes vibrations. |